# Energy Monitoring

INTRODUCTION

This project is an IoT-based energy monitoring system that combines hardware (ESP32, PZEM-004T), backend (Node.js, PostgreSQL), frontend (HTML, CSS, JavaScript), and database technologies to monitor, store, and visualize energy data. The system enables real-time energy tracking, historical data analysis, and export functionalities.

1. Purpose

The project aims to:

- Monitor electrical parameters (voltage, current, power, frequency, power factor, energy).

- Store data for analysis and reporting.

- Provide a user-friendly dashboard for real-time monitoring and historical data visualization.

- Allow users to download data and configure the system.

2. Components

#### Hardware

- ESP32:

  - Connects to the Wi-Fi network to transmit energy data.

  - Runs an HTTP server to serve live energy data in JSON format.

- PZEM-004T:

  - Measures energy parameters such as voltage, current, power, frequency, and power factor.

  - Connects to ESP32 via hardware serial communication.

#### Current Coil

#### Software

- Node.js Backend:

  - Handles API requests and responses.

  - Fetches live data from ESP32 and stores it in a PostgreSQL database.

  - Provides endpoints for retrieving filtered data, resetting Wi-Fi, and downloading data.

- PostgreSQL Database:

  - Stores energy monitoring data, including voltage, current, power, frequency, power factor, energy (kWh), and timestamps.

  - Allows querying of historical data based on filters.

#### Frontend Dashboard

- Built with HTML, CSS, and JavaScript.

- Features a live data display, historical data filters, and graph visualization using Chart.js.

- Provides options to reset graphs, filter data, and download results.

### 3. Functionality

#### Data Monitoring

- Live Data:

  - The ESP32 collects energy data from the PZEM-004T sensor and serves it via an HTTP endpoint.

  - The data includes voltage, current, power, frequency, power factor, and total energy (kWh).

- Historical Data:

  - The backend fetches historical data from PostgreSQL.

  - Users can filter data by date range or specific time intervals.

#### Visualization

- The frontend renders graphs for voltage, current, power, power factor, and energy consumption.

- Dynamic filters allow users to adjust the data range.

#### Data Export

- Historical energy data can be downloaded as an Excel file for offline analysis.

## Energy Monitoring Graphs

### 4. Architecture

- ESP32 (IoT Device):

  - Collects energy data.

  - Serves live data via an HTTP server.

  - Manages Wi-Fi credentials with WiFiManager.

- Backend (Node.js):

  - Acts as an API server.

  - Fetches live data from ESP32.

  - Stores and retrieves data from PostgreSQL.

- Frontend (Dashboard):

  - Displays real-time and historical energy data.

  - Provides graph-based visualization and interactive filters.

- Database (PostgreSQL):

  - Stores energy data for long-term analysis.

### 5. Features

- Real-Time Monitoring:

  - Live data updates every second on the dashboard.

- Graph-Based Visualization:

  - Displays energy trends with graphs for voltage, current, power, power factor, and energy (kWh).

- Data Filters:

  - Users can filter data by date range or specific time intervals.

- Data Export:

  - Historical data can be downloaded as an Excel file.

- Wi-Fi Management:

  - Users can reset or update Wi-Fi credentials for the ESP32.

- Responsive Design:

  - The dashboard is styled for a clean and user-friendly experience.

### 6. Technical Implementation

#### ESP32 Code

- Configures a static IP and sets up Wi-Fi using WiFiManager.

- Communicates with the PZEM-004T sensor to collect energy data.

- Serves /live-data endpoint with JSON responses for voltage, current, power, frequency, power factor, and energy (kWh).

#### Backend API

- Technologies: Node.js, Express, PostgreSQL.

- Endpoints:

  - /api/live-data: Fetches live energy data from ESP32 and stores it in the database.

  - /api/graph-data: Retrieves filtered historical data.

  - /api/reset-wifi and /api/save-wifi: Manages ESP32 Wi-Fi credentials.

  - /api/download-data: Generates an Excel file with energy data for a specified range.

#### Database Schema

- Table: energy_monitoring

- Fields:

  - voltage, current, power, frequency, power_factor, kwh, timestamp.

#### Frontend

- HTML: Structure for the energy monitoring dashboard.

- CSS: Provides styling for the dashboard, ensuring responsiveness and aesthetics.

- JavaScript:

  - Fetches live and historical data.

  - Updates graphs dynamically using Chart.js.

  - Handles user interactions like data filtering, Wi-Fi reset, and data export.

### 7. Deployment

- Frontend:

  - Hosted on Nginx at /var/www/iot.sunware.in.

  - Accessible via http://iot.sunware.in.

- Backend:

  - Runs on port 3000, proxied via Nginx.

- Database:

  - PostgreSQL hosted locally with user authentication.

### 8. Advantages

- Real-Time Monitoring:

  - Immediate insights into energy consumption and efficiency.

- Historical Analysis:

  - Data storage allows for trend analysis and optimization.

- User-Friendly Dashboard:

  - Intuitive interface for both technical and non-technical users.

- Customizable Filters:

  - Flexible data querying for specific analysis needs.

- Portable and Scalable:

  - Modular design enables scaling for multiple devices or locations.

### 9. Potential Enhancements

- Alert Mechanisms:

  - Add alerts for abnormal energy usage.

- User Authentication:

  - Secure access to the dashboard and API endpoints.

- Additional Sensors:

  - Integrate more sensors for temperature, humidity, or device-specific energy monitoring.

- Cloud Integration:

  - Push data to cloud services for remote monitoring and analytics.